CONFIDENTIAL

# Product Requirement Document (PRD): Cluster + Infotainment Connectivity Status Bar Logic(RE.OS)

The details contained in this document represent proprietary, confidential information pertaining to products, technologies, and processes from Royal Enfield. All information in this proposal is to be treated in confidence and no duplication, copyright, or patent application is allowed for any of the details from this document/part/product.

| S.No | Version No | Date | Change Description | Author | Reviewed | Approved |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1.0 | 03 Jul 2026 | 1.Figma 2. Connectivity Logic |  |  |  |

## 1. Objective & Scope

### 1.1 Objective

The objective of this document is to define the exact layout rules, input priorities, visual states, and warning toggle behaviors for the connectivity status icons shown in the top bar of the vehicle's digital cluster. The status bar keeps the rider informed about the connection health of their Phone, Bluetooth (BT) headset/audio device, onboard E-sim, and active Wi-Fi networks.

### 1.2 Scope

This document covers:

- The 16-state Master Connection Matrix that governs which icons appear in the status bar slots.

- Highly detailed state, battery, and signal tracking logic for the paired Smartphone (Icon 1).

- Audio and power state logic for the connected Bluetooth Device (Icon 2).

- Signal and positioning health tracking for the integrated E-sim (Icon 3).

- Local Wi-Fi network strength and status indicators (Icon 4).

- Intelligent on-screen toggle timers for multi-alert scenarios.

## 2. Status Bar Icon Slots & Layout Priority

The status bar features four dedicated, sequential icon locations at the top of the interface, numbered from left to right as Icon 1, Icon 2, Icon 3, and Icon 4.

To maintain clean aesthetics and prevent visual gaps, the status bar dynamically shifts icons to the right or left based on which devices are actively paired or connected. The system processes incoming connection states against a 16-row Master Matrix to route the correct parameters to each active slot.

### 2.1 Master Connection Routing Matrix

The table below outlines how the system routes status readouts to the four icon positions depending on whether a Phone (Y/N), Bluetooth Device (Y/N), E-sim (Y/N), or Wi-Fi Network (Y/N) is connected:

| Case | Phone | BT Device | E-sim | Wifi | Icon 1 (Slot 1) | Icon 2 (Slot 2) | Icon 3 (Slot 3) | Icon 4 (Slot 4) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Y | Y | Y | Y | Phone Battery/Signal/GPS | BT Audio & Battery | E-sim Signal/GPS | Wifi Connected |
| 2 | Y | N | Y | Y | Phone Battery/Signal/GPS | Hidden (Empty) | E-sim Signal/GPS | Wifi Connected |
| 3 | N | Y | Y | Y | Hidden (Empty) | BT Audio & Battery | E-sim Signal/GPS | Wifi Connected |
| 4 | Y | Y | Y | N | Phone Battery/Signal/GPS | BT Audio & Battery | E-sim Signal/GPS | Hidden (Empty) |
| 5 | N | N | Y | Y | Hidden (Empty) | Hidden (Empty) | E-sim Signal/GPS | Wifi Connected |
| 6 | Y | N | Y | N | Phone Battery/Signal/GPS | Hidden (Empty) | E-sim Signal/GPS | Hidden (Empty) |
| 7 | N | Y | Y | N | Hidden (Empty) | BT Audio & Battery | E-sim Signal/GPS | Hidden (Empty) |
| 8 | N | N | Y | N | Hidden (Empty) | Hidden (Empty) | E-sim Signal/GPS | Hidden (Empty) |
| 9 | Y | Y | N | Y | Phone Battery/Signal/GPS | BT Audio & Battery | Hidden (Empty) | Wifi Connected |
| 10 | Y | N | N | Y | Phone Battery/Signal/GPS | Hidden (Empty) | Hidden (Empty) | Wifi Connected |
| 11 | N | Y | N | Y | Hidden (Empty) | BT Audio & Battery | Hidden (Empty) | Wifi Connected |
| 12 | Y | Y | N | N | Phone Battery/Signal/GPS | BT Audio & Battery | Hidden (Empty) | Hidden (Empty) |
| 13 | N | N | N | Y | Hidden (Empty) | Hidden (Empty) | Hidden (Empty) | Wifi Connected |
| 14 | Y | N | N | N | Phone Battery/Signal/GPS | Hidden (Empty) | Hidden (Empty) | Hidden (Empty) |
| 15 | N | Y | N | N | Hidden (Empty) | BT Audio & Battery | Hidden (Empty) | Hidden (Empty) |
| 16 | N | N | N | N | Hidden (Empty) | Hidden (Empty) | Hidden (Empty) | Hidden (Empty) |

## 3. Smartphone Icon Logic (Icon 1)

When a mobile phone is paired and connected via Bluetooth or Wi-Fi projection, the system monitors three distinct parameters: Mobile Battery Level, Cellular Signal Strength, and GPS Signal Health.

### 3.1 Warning Prioritization and Toggling Rules

When multiple conditions are met (such as a low battery along with a low cellular signal), the system cannot fit all warning symbols in a single status bar slot simultaneously.

To solve this, the system uses an Alternating Toggle Timer set to exactly 3 seconds. The system cycles through the active warning states, displaying each icon individually before moving to the next.

### 3.2 Phone Status Parameter Matrix

The following logic rules govern what the rider sees in the Icon 1 position based on real-time device telematics:

| Phone Conn. | Battery % Range | Low Cell Signal? | Low GPS? | Action & Icon Visual State |
| --- | --- | --- | --- | --- |
| No | N/A | N/A | N/A | Icon Hidden. No phone status indicator is displayed. |
| Yes | 80% to 100% | No | No | Full Battery Icon. Solid green or standard white outline battery, indicating a full charge. |
| Yes | 80% to 100% | Yes | No | Low Cell Signal / No Cell Signal Warning. Displays the low cellular bar icon. |
| Yes | 80% to 100% | No | Yes | Low GPS Strength Warning. Displays the satellite warning icon. |
| Yes | 80% to 100% | Yes | Yes | Dual Alert Toggle. Screen alternates every 3 seconds between the Low Cell Signal icon and the Low GPS Strength icon. |
| Yes | 40% to 79% | No | No | 2/3 Battery Icon. Shows a partially depleted battery outline. |
| Yes | 40% to 79% | Yes | No | Low Cell Signal Warning. Displays the low cellular bar icon. |
| Yes | 40% to 79% | No | Yes | Low GPS Strength Warning. Displays the satellite warning icon. |
| Yes | 40% to 79% | Yes | Yes | Dual Alert Toggle. Alternates every 3 seconds between the Low Cell Signal and Low GPS Strength icons. |
| Yes | 20% to 39% | No | No | 1/3 Battery Icon (Color Change). Displays a single charge bar inside the battery. The icon color transitions to Yellow or Amber to indicate low charge. |
| Yes | 20% to 39% | Yes | No | Low Cell Signal Warning. Displays the low cellular bar icon. |
| Yes | 20% to 39% | No | Yes | Low GPS Strength Warning. Displays the satellite warning icon. |
| Yes | 20% to 39% | Yes | Yes | Dual Alert Toggle. Alternates every 3 seconds between the Low Cell Signal and Low GPS Strength icons. |
| Yes | 0% to 19% | No | No | Critical Battery Icon (Color Change). Displays a nearly empty battery outline. The icon color transitions to Red to indicate critical charge. |
| Yes | 0% to 19% | Yes | No | Dual Critical Toggle. Alternates every 3 seconds between the Red Critical Battery icon and the Low Cell Signal icon. |
| Yes | 0% to 19% | No | Yes | Dual Critical Toggle. Alternates every 3 seconds between the Red Critical Battery icon and the Low GPS Strength icon. |
| Yes | 0% to 19% | Yes | Yes | Triple Critical Toggle. Alternates every 3 seconds across three states: Red Critical Battery Low Cell Signal Low GPS Strength. |

## 4. Bluetooth Device / Headset Logic (Icon 2)

When a Bluetooth audio device (such as a helmet headset, wireless intercom, or mobile earpiece) is paired to the cluster, its status is displayed in the second active slot.

### 4.1 Connection and Display Rules

- No Device Connected: The Bluetooth icon is hidden.

- Device Connected: The system displays a headset/intercom icon paired with a vertical or horizontal battery level indicator representing the remote device's remaining charge.

- Low Battery Warning: If the connected Bluetooth device reports a battery level below 20%, the battery portion of the icon changes color to Yellow/Red to alert the rider to charge their headset.

## 5. Onboard E-sim Icon Logic (Icon 3)

The vehicle contains an integrated E-sim to provide continuous cellular telemetry back to the fleet and user cloud applications. The health of this connection is reflected in the third active status bar slot.

### 5.1 Parameter and Status Logic

Similar to the smartphone monitoring system, the E-sim tracking loop evaluates both Telematic Cell Signal and Onboard GPS Strength. Because the E-sim runs directly off the bike's main battery system, it does not display an individual battery status.

| Low Cell Signal? | Low GPS Strength? | Action & Icon Visual State |
| --- | --- | --- |
| No | No | Full Connection Icon. Displays standard signal bars indicating a strong cellular network and GPS connection. |
| Yes | No | Low Cell / No Cell Signal Warning. Displays the low cellular signal bar icon with an active "no service" strike-through. |
| No | Yes | Low GPS Strength Warning. Displays the warning satellite icon. |
| Yes | Yes | Dual Warning Toggle. The slot alternates every 3 seconds between the Low Cell Signal icon and the Low GPS Strength icon. |

## 6. Wi-Fi Connectivity Logic (Icon 4)

When the vehicle is parked near a garage or service station and connects to a local Wi-Fi network for map downloads or software updates, the status of this connection is displayed in the final status bar slot.

### 6.1 Wi-Fi Display States

- Disconnected / Off: The Wi-Fi icon is completely hidden.

- Connected: Displays the standard Wi-Fi wave icon. The number of active arcs in the icon corresponds directly to the received signal strength indicator (RSSI):

  - Strong Signal: 3 arcs displayed.

  - Medium Signal: 2 arcs displayed.

  - Weak Signal: 1 arc displayed.

7. Figma URL

https://www.figma.com/design/3Oiz9DpEozXaeIvr0ozgfg/Royal-Enfield---External?node-id=75468-360806&t=jxxayUhSPihBWujL-0